|  |  |  |  |  |  |  |  | Contract Number: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Assessed by: | Name: |  |  | Position: |  |  |  |  | Date: |  |  |
|  | Signed: |  |  |  |  |  |  |  |  |  |  |
| Description Of Work: | Use of Wadkin crosscut saw for cutting timber and dado work. |  |  |  |  |  |  |  |  |  |  |
| Task / Job Component | ## Hazard |  | Persons at risk |  |  | Risk Rating L/M/H | Controls / Precautions to Reduce Risk |  |  |  | Residual Risk Rating L/M/H |
| Use of Wadkin crosscut saw for cutting timber and dado work. | Cuts, amputation |  | Operator, persons nearby |  |  | H | - Only trained persons permitted to operate machine. - Check that blades are not worn or damaged (checked daily). - Ensure hands are kept out of the hatched area. - Extend barrier to prevent access by others when saw is in use. - Push stick to be used for short lengths of wood. - Multiple short cuts require use of spacer block and clamps. - Machine to be isolated and locked off after use. - Machine is maintained annually. - Safe use instructions are displayed adjacent to machine. |  |  |  | M |
|  | Noise |  | Operator, persons nearby |  |  | M | - Hearing protection to be worn at all times. |  |  |  | L |
|  | Dust |  | Operator, persons nearby |  |  | M | - Ensure LEV is switched on before starting work. - Sundstrom P3 cartridge mask to be worn. - Operator to be Face Fit Tested for mask. |  |  |  | L |
|  | Flying particles causing eye injury |  | Operator, persons nearby |  |  | H | - Goggles to BSEN 166 B to be worn at all times. |  |  |  | L |
|  | Slips, trips |  | Operator, persons nearby |  |  | M | - Ensure area around machine is tidy and free from materials and excess sawdust. - Regularly vacuum up sawdust on floor if required during use. - Anti-slip paint applied to floor in workshop. |  |  |  | L |

# Likelihood

How often could the hazard occur? Consider the task, frequency, duration, method of work, employees involved.

# Severity

# How serious would the hazard’s effects be if

realised? Consider the type of hazard, biological, ergonomic, physical and chemical.

# Risk = Likelihood x Severity

E.g. Likelihood (4) X Severity (3) = 12 HIGH RISK